USCG PIC (PILOT IN COMMAND) REQUIREMENTS REFERENCE

Based on COMDTINST M3710.1I and AIRSTAMIAINST M3710.1C

## 1. AUTHORITY & FLIGHT CLEARANCE

The PIC has clearance authority to depart on a mission

PIC is responsible for the safe conduct of any individual mission

PIC must contact Operations Officer if:

Conducting a precautionary landing (including at home unit)

Aborting a mission

Requesting to cancel a flight for safety concerns (weather, aircraft malfunction, damage)

## 2. DIVERSION AUTHORITY

PICs have authority to divert to higher priority missions, subject to:

Notify Operations Center ASAP when diverted by Southeast District or CTU with TACON

Ensure aircraft and aircrew are fully capable of safely completing the diverted mission

When deployed, notify OPCON/TACON as soon as practical after deviation from assigned tasking

## 3. PILOT SEAT ASSIGNMENT REQUIREMENTS

During dual pilot flights, the PIC will occupy the left seat

Both pilot seats must be occupied for aircraft operations requiring dual crew

## 4. SCHEDULING & DELAYS

PICs shall make every effort to meet assigned takeoff times

Notify Operations Center for delays greater than 30 minutes

Annotate reason for delay in ALMIS event status upon return

Any request to cancel must be approved by Operations Officer

Cannot deviate from scheduled missions without approval for other than exigent circumstances

## 5. FUEL MANAGEMENT

Must maintain established minimum landing fuel reserves

If landing with less than minimum fuel reserves, advise Operations Officer ASAP

Ensure adequate fuel for safe mission completion before accepting flight

## 6. FLIGHT PLANNING & CLEARANCES

Foreign Clearance Responsibilities:

PICs are responsible for obtaining foreign clearances for scheduled missions

Prior to short-notice emergency missions requiring non-U.S. territory entry, ensure clearances are properly coordinated

Coordinate with Operations Officer when required

Passenger Authority:

Obtain approval from OPS to carry passengers (unless already annotated on flight schedule)

## 7. MISSION EMPLOYMENT & EMPLOYMENT CODES

PIC shall ensure proper employment category is used in ALMIS for mission flown

Communicate with Operations Officer to discuss risk and training plan when:

Ceilings/visibility are less than 500-1

Sustained winds greater than 25 knots

Gust spread greater than 15 knots

## 8. AIRCRAFT OPERATIONS & MAINTENANCE

Malfunction/Damage Procedures:

Report nature of malfunction/damage to ODO/Operations Officer, then Engineering Officer

Notify Flight Safety Officer after Engineering evaluation

Maintenance Release Authority:

If aboard ship with TACON assigned, PIC may act as maintenance release authority for routine maintenance and minor scheduled discrepancies (with prior written authorization)

Functional Check Flights (FCF):

Brief CO prior to conducting FCF with specific required briefing content

Bird Strike Procedures:

If PIC experiences/suspects airframe bird strike, follow Flight Manual emergency procedures

Conduct appropriate post-flight procedures and inspections

## 9. EMERGENCY & WEATHER OPERATIONS

Hot Refueling:

PICs shall comply at all times with aircraft Flight Manual procedures

Follow procedures in TIMI 3-01 Aircraft Fueling and Defueling Operations

Sterile Cockpit:

Ensure sterile cockpit between OPF and coastline due to high-density traffic and mid-air collision potential

Thermal Stress Awareness:

Familiarize with Thermal Stress Plan in AIRSTAMIAINST 5100.1

Brief Operations Officer when outside air temperature exceeds specified thresholds

## 10. TRAINING & READY POSTURE

Ready aircraft training outside authorized areas requires notification to Operations Officer

Ready-in-the-air may be switched to ready-recallable at PIC discretion for crew/training requirements

ODO must be notified prior to launch if changing posture

If designated ready-recallable while airborne, adjust aircraft posture to comply ASAP

Limit simulated emergencies to discussion only unless approved by Operations Officer

## 11. AIRSPACE OPERATIONS & AIR DEFENSE

Monitor latitude/longitude of aircraft in relation to ADIZ

Squawk appropriate Mode-3 code: unit-assigned law enforcement code or assigned discrete code

Contact ODO, Southeast District, or Sector Controller if unable to coordinate initial call to HUNTRESS/AMOC

For scheduled LE patrols:

Contact controlling agency to determine availability and coordinate deconfliction

Check in with controlling agency and monitor assigned frequency when area is active

If unable to coordinate deconfliction or safe margin cannot be maintained, report to appropriate authority

## 12. LAW ENFORCEMENT OPERATIONS

Use encrypted communications for all LE missions

If encrypted communications unavailable, contact OPCON for guidance on continuing mission

Provide post-flight report via email with activity descriptions, locations, etc.

Maintain responsibility to obtain proper clearance (Southeast District does NOT relieve PIC of this)

## 13. ADS-B & MODE S OPERATIONS

ADS-B Out Off decision shall be made on deck prior to launch

Contact Operations Officer to discuss risk vs. gain if in doubt

For Mode S, contact Operations Officer and discuss risks vs. gain prior to turning off

Mode S is inherent TCAS II capability - consider collision avoidance implications

## 14. MEDICAL OPERATIONS (MEDEVAC)

Final Decision Authority:

Final decision to conduct MEDEVAC operation rests with PIC (with Flight Surgeon/SMC concurrence)

Personnel Protective Equipment:

Give consideration to appropriate PPE during MEDEVAC operations

Hospital Selection:

SMC typically recommends hospital, but PIC may provide alternate location if adequate margin of safety cannot be maintained

## 15. SECURITY REQUIREMENTS

Aircraft Security Away from Home Station:

PIC is responsible for security of aircraft and contents while away from home station

Aircraft shall always be locked when parked away from Air Station

Store all sensitive/classified materials in aircraft safe if installed

Follow Emergency Action and Destruction Plan per applicable instructions

Public Events & Flying Shows:

Security of aircraft is paramount and is PIC responsibility

Consider: high-volume crowds, audience age, likelihood of damage based on demeanor

Brief coordination with External Affairs staff prior to event

Briefing shall cover: op hazards, expected route, ground coordination contact, airspace/ATC requirements, minimum safe altitude/airspeed

## 16. DEPLOYED OPERATIONS

Pre-Deployment:

Schedule pre-deployment brief with Intelligence Division

Confirm APACS and foreign country clearances are complete

Obtain and complete pre-deployment checklist

Crew Accommodations:

Ensure all genders have separate accommodations to maximum extent possible

Ensure all crew members have individual rooms to maximum extent possible

Notify appropriate authority if facilities unavailable

Post-Deployment:

Complete post-deployment debrief with Law Enforcement Division (or delegate to junior deployed aviator)

Debrief must be completed within one week of return

## 17. FERRY FLIGHTS

Brief Operations Officer on conduct and route of flight

Meet with Engineering Department at least 48 hours prior to departure

## 18. PUBLIC AFFAIRS & OPERATIONAL SUPPORT

Consider requesting cover anytime concern exists over mission execution and safety

Cover provides adequate communications and/or navigation capability

## 19. CRITICAL OPERATIONAL AWARENESS

Restricted Areas:

Be aware of: Key Deer National Wildlife Refuge, Great White Heron National Refuge

Be aware of: Biscayne Bay and Everglades National Parks when transiting or prosecuting missions

## QUICK REFERENCE: PIC DECISION AUTHORITIES

| Decision/Action | Authority/Approval Required | Key Conditions |
| --- | --- | --- |
| Divert Aircraft | PIC Authority | Aircraft/crew capable; notify Ops Center |
| Precautionary Landing | PIC Authority | Must notify Ops Officer |
| Cancel Flight | Operations Officer Approval | Safety concerns |
| Land with Low Fuel | PIC Authority | Advise Ops Officer ASAP |
| Foreign Territory Entry | PIC Responsibility | Ensure clearances coordinated |
| Carry Passengers | OPS Approval | Unless on approved schedule |
| Change Employment Code | PIC (notify Ops) | Reflects actual mission |
| Hot Refuel | PIC Authority | Follow Flight Manual & TIMI 3-01 |
| MEDEVAC Conduct | PIC Final Decision | With Flight Surgeon/SMC concurrence |
| ADS-B Out Off | PIC (on deck) | Discuss with Ops if in doubt |
| Mode S Off | Discuss with Ops Officer | Risk vs. gain decision |
| Training Outside Authorized Area | Notify Ops Officer | Before launch |
| FCF Conduct | PIC Authority | Brief CO prior to flight |
| Cover Request | PIC Discretion | Safety/mission execution concerns |
| LE Patrol Coordination | PIC Responsibility | Coordinate with controlling agency |